LAMBRETTA CLASS

Production EVO Class

For Lambretta models - Any Series 3, GP, SX , LI model allowed. Italian, Indian, or Spanish of origin.

Bodywork

Standard body work with no trimming of panels except for hole in the side panel for carburettor and small cut to rear of r/h running board for tail pipe of exhaust.

Centre stand and splash plate must be removed.

Metal or Fibreglass panel work is allowed, Fibreglass legsheilds are not allowed.

No dropped handlebars allowed but any series 3 standard headset can be used.

No light lenses front or rear and headlight opening must be blanked off. Rear light housing must be in position.

19mm Ball end brake and clutch levers must be fitted.

Suspension

Any Lambretta specific rear shock that is commercially available.

Must be in standard position. e.g. no movement of original rear suspension

fixing positions.

Front forks as original internally sprung with uprated springs and bushes.

Any commercially available external front dampers allowed.

Steering dampers are not allowed.

Braking

Rear brake as original with commercially available shoes. Rear brake pedal in standard position.

Front brake as original drum or inboard disc. Conversion to hydraulically operated piston in allowed.

External single hydraulic disc ScootRS type with drum style hub.

Single outboard only is permitted, No anti dive allowed.

Engine

Standard 150 engine casing with standard mag flange.

No welding to either and no pack plated between mag housing and engine casing.

No material may be removed from the mag housing or engine casing.

No matching of transfers allowed.

Bump stop to remain intact.

Rubber engine mounts may be uprated.

Side casing with kick-start removed and welded up, or pedal removed and rubber cap secured over exposed kick-start shaft end.

No packing plate allowed between side casing and engine.

Type specific Raypeed RB20 performance kit and cylinder head in standard form with no polishing or tuning allowed.

Standard Vertex or Wossner piston can be used. No machining or modifications to piston windows allowed.

Standard inlet with standard reed block and petals.

Any 58mm Stroke crankshaft (NOT FULL CIRCLE) with 116mm con-rod. Big end pin can be tig welded in position.

Road stator plate for electrics with flywheel in use. Standard weight, intermediate or lightened flywheel may be used.

No advance / retard mechanisms for ignition timing may be used.

Ignition timing must be fixed.

Original fan cowling and cylinder cowling must be in place for cooling, Centre of fan cowling may be removed. 
No additional cooling scoops or ram cooling allowed.

Filler and drain plugs to be lock wired.

Exhaust

Type specific Franspeed RB race pipe specially designed for this class.

Carburettor

Carburettor used must be from approved specified list. Dellorto VHSH 30 round slide, Dellorto VHSA 30 flat slide. No polishing, No Power jet or modifications to standard carb. No force induction, or ram pipes to be fitted to carb.

Only the use of standard Vetronite reed petals, as supplied with the kit is permitted.

Clutch

Any commercially available standard or Cassette clutch allowed, 4,5,6 or 7 plates with cable action as original. No chaincase packer or side casing modifications allowed.

Gearbox & Drive

Must be as per original Specification type specific. (Sil GP 200)

Front & rear sprockets sizes may be altered to suit rider and circuit. 
Only commercially available sprockets may be used.

Uprated chain tensioner is allowed.

Tyres

Type specific: Tyres brands must only be used on Lambretta split or tubeless rims.  
Sava MC20 ( Wet ),  Sava MC 31 ( Dry ). 
Continental Race ( Dry ) and Continental Move ( Wet ).

Gauges & Controls

Only a rev counter is allowed, no other monitoring devices may be fitted.

Fuel

Commercially available pumped fuel or Avgas with all taxes paid.

Fuel tap must be clearly marked Fuel on – off in relevant position.

SPECIAL NOTE…

As with all classes in the BSSO any engine where performance is deemed non-compliant with the expectations for the class it is running in, is liable to have the engine stripped and checked by the technical committee.

It is a condition of entry that all competitors will be issued with a set of Sponsors Logo Stickers, which are to be fitted to both sides of their machine during all races in the 2016  championship. Sticker sets will be available prior to the commencement of the season.

VESPA CLASS

Production EVO Class

For Vespa Models - Any Vespa PX, also Small Frame, Manufacture origin may be Italian, Indian, or Spanish models, are allowed.

Bodywork

VESPA - any model PX, and Small Frame Vespa

Body work as standard in appearance

No dropped handlebars

Metal panels or fibreglass panels may be used, with the exception of legshields

No light lenses front or rear and headlight opening must be blanked off. 
Rear light housing must be in position.

19mm Ball end levers must be fitted to the machine

Ignition switch to be clearly marked ON – OFF.

Self-closing throttle must be used.

Clearly marked race numbers as ACU sizes and guidelines.
White background with Black numbers

Suspension

Forks must be standard in appearance, uprated front damper is allowed

Hydraulic front disc brake is allowed on safety grounds

The use of steering dampers is not allowed

The rear suspension unit to be fitted in the standard position, but can be uprated for performance and safety

Braking

The rear brake pedal must be standard, with cable operation and in its original position. No rear set brakes.

Engine

All engine cowlings must be of standard design,

The centre stand can be removed to allow ground clearance

Filler and drain plugs must be wired to ensure they don’t come loose

The cylinder is to receive no tuning or welding

The electronic ignition system must be standard and static timed 
No varitronic system or additional advance and retard system is allowed

Standard flywheel, intermediate weight flywheel or lightened flywheel may be used

The crankcase must be standard, no welding of any description is allowed, either internal or external

No matching of transfer ports

Engine mounts may be uprated.

Each Malossi performance kit and cylinder head used to be in standard form with no polishing or tuning allowed.

It is a condition of entry that all competitors will be issued with a set of Sponsors Logo Stickers, which are to be fitted to both sides of their machine during all races in the 2016  championship. Sticker sets will be available prior to the commencement of the season.

The engine should be type specific using the following Specification:

PX200 - Type specific

PX 200 using Malossi 210 MHR cylinder kit & cylinder head

A competition crankshaft 57mm

Dellorto 30mm PHBH Standard induction carb kit and manifold

Franspeed L/H exhaust

PX125/150 - Type specific

PX 125/150 using Malossi 166 cylinder kit & cylinder head

A competition crankshaft 57mm

Dellorto 30mm PHBH Standard induction carb kit and manifold

Franspeed L/H exhaust

Small Frame 135 - Type specific

Small frame using Malossi 135 MHR cylinder kit & cylinder head

51mm competition crankshaft

Dellorto 25mm PHBL Standard induction carb kit and manifold

NFK Signature race exhaust

Exhaust

Type specific - Franspeed race pipe specially designed for the PX125 and PX200.

Type specific - NFK Signature race system designed for Vespa 135 small frame.

Carburettor

Carburettor used must be from approved specified list. Dellorto PHBH 30mm,

Dellorto PHBL 25mm. No polishing, No Power jet or modifications to standard carb.

No force induction, or ram pipes to be fitted to carb.

Clutch

The Cosa type clutch unit can be used for the PX200 model

Gearbox & Drive

The PX200 gearbox can be modified to take a T5 4th gear and the use of a 22t clutch drive plate, PX125/150 gearbox can be modified to a 21 or 22 tooth clutch drive plate.

Tyres

Type specific Tyres brands must only be used on Vespa split or tubeless rims.  
Sava MC20 ( Wet ),  Sava MC 31 ( Dry ). 
Continental Race ( Dry ) and Continental Move ( Wet ).

Gauges & Controls

Only a rev counter is allowed, no other monitoring devices may be fitted.

Fuel

Commercially available pumped fuel or Avgas with all taxes paid.

Fuel tap must be clearly marked Fuel on – off in relevant position.

SPECIAL NOTE…

As with all classes in the BSSO any engine where performance is deemed non-compliant with the expectations for the class it is running in, is liable to have the engine stripped and checked by the technical committee.

It is a condition of entry that all competitors will be issued with a set of Sponsors Logo Stickers, which are to be fitted to both sides of their machine during all races in the 2016  championship. Sticker sets will be available prior to the commencement of the season.